Policy

Personal Protective Equipment

FSC-POL-30-001b V1-0 EN

| Title: | Personal Protective Equipment |
| --- | --- |
| Dates: | Approval date: 15 March 2022 |
| Timeframes: | Transition period: Until the International Generic Indicators for Highly Hazardous Pesticides have been incorporated into Forests Stewardship Standards. |
| Contact for comments: | FSC International – Performance and Standards Unit Adenauerallee 134 53113 Bonn Germany Phone: +49 -(0)228 -36766 -0 Fax: +49 -(0)228 -36766 -65 Email : psu@fsc.org |

### Version control

| Publication date: | 14 April 2023 |  |
| --- | --- | --- |
| Effective date: | 1 July 2023 |  |
| Version | Description | Date |
| V1-0 | Initial version, approved by the FSC Board of Directors at their 91st meeting as part of the International Generic Indicators for Highly Hazardous Pesticides | 15 March 2022 |

® 2023 Forest Stewardship Council, A.C. All Rights Reserved

FSC® F000100

You may not distribute, modify, transmit, reuse, reproduce, re-post or use the copyrighted materials from this document for public or commercial purposes, without the express written consent of the publisher. You are hereby authorized to view, download, print and distribute individual pages from this document subject for informational purposes only.

# Introduction

The objective of this document is to provide information to support Standard Developers to determine the personal protective equipment needs for the use of a highly hazardous pesticide (HHP), considering the hazard group, hazard criterion and subset the HHP belongs to, that shall be considered when adapting International Generic Indicator 10.7.15 in FSC-STD-60-004 V2-1 International Generic Indicators.

Contents

Introduction	3

Scope	5

References	5

1.	Personal protective equipment	6

# Scope

This document specifies requirements for Standard Development Groups developing or revising International Generic Indicators for the use of highly hazardous pesticides into their Forest Stewardship Standards.

All aspects of this document are considered normative, including the scope, effective and
validity dates, references, terms and definitions, footnotes, graphics, tables and annexes,
unless otherwise stated.

Notes, information/guidance boxes and examples are not considered normative.

# References

The following referenced documents are relevant for the application of this document.

For references without a version number, the latest version of the referenced document (including any amendments) applies:

| FSC-POL-30-001 | FSC Pesticides Policy |
| --- | --- |
| FSC-POL-30-001a | FSC Lists of Highly Hazardous Pesticides |
| FSC-STD-60-004 | International Generic Indicators |

# Personal protective equipment

INSTRUCTIONS FOR STANDARD DEVELOPERS:

This table provides information for determining the personal protective equipment needs for the use of a highly hazardous pesticide (HHP), considering the hazard group, hazard criterion and subset the HHP belongs to, that shall be considered when adapting the International Generic Indicator 10.7.15.

Standard Developers shall ensure that requirements for personal protective equipment in the national indicators, beside meeting the requirements in national laws, regulations and product labels, meet or exceed the requirements specified in this table.

In all cases, the personal protective equipment shall be appropriate to mitigate the exposure risk associated with the specific HHP, e.g. respirators to mitigate inhalation risk or gloves and overalls to mitigate dermal absorption risk.

The column “Subset of chemical pesticides” provides a partial listing of chemicals in the given hazard group. For a complete list of chemicals in any hazard group, Standard Developers shall refer to the most recent version of FSC-POL-30-001a FSC Lists of highly hazardous pesticides.

The columns “Hazard group” and “Hazard criterion” are as described in FSC-POL-30-001.

The sources of the information provided in the columns “Personal protective equipment’ and “Classification” are FAO/WHO International Code of Conduct on Pesticide Management: Guidelines for Personal Protection when Handling and Applying Pesticides, 2020.

| Subset of chemical pesticides | Hazard group | Hazard criterion and GHS classification | Personal protective equipment | Classification |
| --- | --- | --- | --- | --- |
| Organochlorines/ chlorinated hydrocarbons (DDT, Endosulfan, Atrazine, Vinclozolin, Chlordane, Lindane, Hexachlorobenzene,) | Relevant International Agreements or conventions | - Relevant International Agreements or Conventions GHS05 DANGER H290, H314, H318 GHS06 DANGER H300, H301 H310, H311, H330, H331, GHS07 WARNING Ozone depleting H420 | 1.Butyl rubber gloves | EN 374:2016 |
| Chemical pesticides listed in the Annex III of the Rotterdam Convention on the Prior Informed Consent Procedure (Benomyl, Captafol, Carbofuran, DDT, DNOC, DNOC ammonium salt, DNOC potassium salt, DNOC sodium salt, Ethylene dibromide, Ethylene dichloride, Ethylene oxide, Fluoroacetamide, , Hexachlorobenzene, Hexachlorocyclohexane, Lindane, Mercury, , Monocrotophos, Paraquat dichloride, Parathion-methyl, PCP, Thiram ) |  |  | 2.Type 3 protective clothing (liquid tight) | EN 14605:2005 |
| Methyl bromide |  |  | Type 4 protective clothing (spray tight) | EN 14605:2005 |
|  |  |  | Type 5 protective clothing (airborne particles) | BS EN ISO 13982:2004 |
|  |  |  | Type 6 protective clothing (chemical splash) | EN 13034 |
|  |  |  | 3.Safety boots | EN 345:1993 EN ISO 20345 |
|  |  |  | 4.Full face respirators | EN 136 |
|  |  |  | Full face respirators for vapours and gases. | EN 141:2000 |
|  |  |  | 5. Particulate air filters for respirators. | P95, P99, P100 |
|  |  |  | 6. Apron | EN 467:1995 |
| Bupyridyls (Paraquat, Diquat, Diquat dibromide) | Acute toxicity | - Acute toxicity to mammals and birds GHS05 DANGER H314, H318 GHS06 DANGER H330, H301 H310, H311 H330, H331 GHS07 WARNING H302, H312 H332, H315 H317, H319 | 1.Chemically resistant nitrile gloves | EN 374:2016 |
|  |  |  | 2.Type 3 and type 4 protective clothing | EN14605:2005 |
|  |  |  | 3.Safety boots | EN 345:1993 EN ISO 20345 |
|  |  |  | 4.Face and Eye protection (safety goggles) | EN 166: 2001 |
|  |  |  | 5.Half face respirators | EN 140, EN 149 EN 143: 2000 |
|  |  |  | 6.Particulate air filters for respirators | R95, R99, R100 |
|  |  |  | 7.Apron | EN 467: 1995 |
| Neonicotinoids (Acetamiprid, Imidacloprid, Thiacloprid) | Acute toxicity | - Acute toxicity to mammals and birds GHS05 DANGER H314, H318 GHS06 DANGER H300,H301 H310,H311 H330, H331 GHS07 WARNING H302, H312 H332, H315 H317, H319 | 1.Neoprene glove | EN 374:2016 |
|  |  |  | 2.Type 3 and Type 4 protective clothing | EN14605:2005 |
|  |  |  | Type 5 protective clothing | BS EN ISO 13982: 2004 |
|  |  |  | 3.Safety boots | BS EN ISO 13982: 2004 EN 345:1993 EN ISO 20345 |
|  |  |  | 4.Face and eye protection | EN 166:2001 |
|  |  |  | 5.Half face respirator | EN 140 EN149 EN143: 2000 |
|  |  |  | 6.Particulate filters for respirators | R95, R99, R100 |
|  | Chronic toxicity | - Mutagenicity to mammals GHS08 DANGER H340, H341 | 7.Apron | EN 467: 1995 |
|  | Environmental toxicity | 8 Persistence in soil or water and soil sorption potential and bio-magnification and bio-accumulation GHS09 WARNING H410, H411 H412, 4413 Environment |  |  |
| Pyrethroids (Cyfluthrin. Cypermethrin, Deltamethrin, Permethrin) Phenoxyalkyl acids Amides (Alachlor, Butachlor, Propachlor) | Acute toxicity | 2 Acute toxicity to mammals and birds GHS05 DANGER H314, H318 GHS06 DANGER H300, H301, H310, H311, H330, H331 GHS07 WARNING H302, H312, H332, H315, H317, H319 | 1.Neoprene gloves/ chemically resistant gloves | EN 374:2016 |
|  |  |  | 2.Type 3 and Type 4 protective clothing | EN14605:2005 |
|  |  |  | Type 5 protective clothing | BS EN ISO 13982: 2004 |
|  |  |  | 3.Safety boots | BS EN ISO 13982: 2004 EN 345:1993 EN ISO 20345 |
|  |  |  | 4.Face and eye protection | EN 166:2001 |
|  |  |  | 5.Half face respirator | EN 140 EN149 EN143: 2000 |
|  |  |  | 6.Particulate filters for respirators | R95, R99, R100 |
|  | Chronic toxicity | 3 Carcinogenicity GHS07 WARNING H335, H336 GHS08 DANGER H334, H350 H350i, H350l | 7.Apron | EN 467: 1995 |
|  |  | 4 Mutagenicity to mammals GHS08 DANGER H340, H341 |  |  |
|  |  | 6 Endocrine Disrupting Chemicals (EDC) GHS08 DANGER H370, H371 H372, H373 |  |  |
| Carbamates Thiocarbamates (Mancozeb, Maneb, Thiram, Molinate) | Chronic toxicity | 3 Carcinogenicity GHS07 WARNING H335, H336 GHS08 DANGER H334, H350 H350i, H350l | 1.Neoprene gloves/ chemically resistant gloves | EN 374:2016 |
|  |  |  | 2.Type 3 and Type 4 protective clothing | EN14605:2005 |
|  |  |  | 3.Safety boots | BS EN ISO 13982: 2004 EN 345:1993 EN ISO 20345 |
|  |  |  | 4.Face and eye protection (safety googles) | EN 166:2001 |
|  |  |  | 5.Half-face respirators | EN 140 EN149 EN143: 2000 |
|  |  |  | 6.Particulate filters for respirators | R95, R99, R100 |
|  | Environmental toxicity | 7 Acute toxicity to aquatic organisms GHS09 WARNING H400 Environment | 7.Apron | EN 467: 1995 |
| Organophosphates (Malathion, Parathion, Diazinon, Fenthion, Dichlorvos, Chlorpyrifos, Ethion) | Acute toxicity | 2 Acute toxicity to mammals and birds GHS05 DANGER H314, H318 GHS06 DANGER H300,H301 H310,H311 H330, H331 GHS07 WARNING H302, H312 H332, H315 H317, H319 | 1.Neoprene gloves/ chemically resistant gloves | EN 374:2016 |
|  |  |  | 2.Type 3 and Type 4 protective clothing | EN14605:2005 |
|  |  |  | Type 5 protective clothing |  |
|  |  |  | 3.Safety boots | EN 345:1993 EN ISO 20345 |
|  |  |  | 4.Face and eye protection | EN 166:2001 |
|  |  |  | 5.Half face respirators | EN 140 EN149 EN143: 2000 |
|  |  |  | 6.Particulate filters for respirators | R95, R99, R100 |
|  | Chronic toxicity | 3 Carcinogenicity GHS07 WARNING H335, H336 GHS08 DANGER H334, H350, H350i, H350l | 7.Apron | EN 467: 1995 |
|  |  | 4 Mutagenicity to mammals GHS08 DANGER H340, H341 |  |  |
|  |  | 5 Developmental and reproductive toxicity GHS08 DANGER H360, H360F H360D, H360FD H361, H361f, H361d, H361fd H362 |  |  |
|  | Chronic toxicity | 6 Endocrine Disrupting Chemicals (EDC) GHS08 DANGER H370, H371 H372, H373 |  |  |
|  | Environmental toxicity | 7 Acute toxicity to aquatic organisms GHS09 WARNING H400 Environment |  |  |
| Carbamates Dithiocarbamates (Metiram, Ziram) | Acute toxicity | 2 Acute toxicity to mammals and birds GHS05 DANGER H314, H318 GHS06 DANGER H300,H301 H310,H311 H330, H331 GHS07 WARNING H302, H312 H332, H315 H317, H319 | 1.Neoprene gloves/ chemically resistant gloves | EN 374:2016 |
|  |  |  | 2.Type 3 and Type 4 protective clothing | EN14605:2005 |
|  |  |  | Type 5 protective clothing |  |
|  |  |  | 3.Safety boots | EN 345:1993 EN ISO 20345 |
|  |  |  | 4.Face and eye protection | EN 166:2001 |
|  |  |  | 5.Half face respirators | EN 140 EN149 EN143: 2000 |
|  |  |  | 6.Particulate filters for respirators | R95, R99, R100 |
|  | Chronic toxicity | 5 Developmental and Reproductive toxicity GHS08 DANGER H360, H360F H360D, H360FD H361, H361f, H361d, H361d H362 | 7.Apron | EN 467: 1995 |
|  | Chronic toxicity | 6 Endocrine Disrupting Chemicals (EDC) GHS08 DANGER H370, H371 H372, H373 |  |  |
|  | Environmental toxicity | 7 Acute toxicity to aquatic organisms GHS09 WARNING H400 Environment |  |  |
| Di-nitro anilines (Fluchloralin, Oryzalin, Pendimethalin, Trifluralin) | Chronic toxicity | 3 Carcinogenicity GHS07 WARNING H335, H336 GHS08 DANGER H334, H350 H350i, H350l | 1.Neoprene gloves/ chemically resistant gloves | EN 374:2016 |
|  |  |  | 2.Type 3 and Type 4 protective clothing | EN 14605: 2005 |
|  |  |  | Type 5 protective clothing |  |
|  |  |  | 3.Safety boots | EN 345:1993 EN ISO 20345 |
|  |  |  | 4.Face and eye protection | EN 166:2001 |
|  |  |  | 5.Half face respirators | EN 140 EN149 EN143: 2000 |
|  |  |  | 6.Particulate filters for respirators | R95, R99, R100 |
|  |  | 6 Endocrine Disrupting Chemicals (EDC) GHS08 DANGER H370, H371 H372, H373 | 7.Apron | EN 467: 1995 |
|  | Environmental toxicity | 8 Persistence in soil or water and soil sorption potential and bio-magnification and bio-accumulation GHS09 WARNING H410, H411 H412, 4413 Environment |  |  |
| Glycines (Glyphosate) | Chronic toxicity | 3 Carcinogenicity GHS07 WARNING H335, H336 GHS08 DANGER H334, H350 H350i, H350l | 1.Neoprene gloves / chemically resistant nitrile gloves | EN 374:2016 |
|  |  |  | 2.Type 3 and Type 4 protective clothing | EN 14605: 2005 |
|  |  |  | 3.Safety boots | EN 345:1993 |
|  |  |  | 4.Face and eye protection | EN 140 EN 149 |
|  |  |  | 5.FPP3 masks | EN 149:2001 |
|  |  |  | 6.Apron | EN 467: 1995 |
| Dioxins | Dioxins | 9 Dioxins (residues/ emissions) GHSO6 DANGER H300, H301, H310, H311 H330, H331 GHS08 DANGER H304, H334, H370, H371 H372, H373 GHS09 WARNING H400, H410, H411, H412, H413 Environment | 1.Butyl rubber gloves | EN 374:2016 |
|  |  |  | 2.Type 3 and Type 4 protective clothing | EN 14605: 2005 |
|  |  |  | Type 5 protective clothing | BS EN ISO 13982: 2004 |
|  |  |  | 3.Safety boots | EN 345:1993 EN ISO 20345 |
|  |  |  | 4.Full face respirators for gases and vapours | EN 136 EN 141:2000 |
|  |  |  | 5.Particulate filters for respirators | P95, P99, P100 |
|  |  |  | 6.Apron | EN 467: 1995 |
| Heavy metals | Heavy Metals | 10 Heavy Metals GHSO6 DANGER H300, H310, H330, H331 GHS08 DANGER H304, H334, H340, H341, H350, H351, H360, H361, H362, H370 H370, H371, H372 GHS09 WARNING H400, H410, H411, H412, H413 Environment | 1.Butyl rubber gloves | EN 374:2016 |
|  |  |  | 2.Type 3 and Type 4 protective clothing | EN 14605: 2005 |
|  |  |  | Type 5 protective clothing | BS EN ISO 13982: 2004 |
|  |  |  | 3.Safety boots | EN 345:1993 EN ISO 20345 |
|  |  |  | 4.Full face respirators for gases and vapours | EN 136 EN 141:2000 |
|  |  |  | 5.Particulate filters for respirators | P95, P99, P100 |
|  |  |  | 6.Apron | EN 467: 1995 |

FSC International – Performance and Standards Unit

Adenauerallee 134

53113 Bonn

Germany

Phone:	+49 -(0)228 -36766 -0

Fax:	+49 -(0)228 -36766 -65

Email :	psu@fsc.org

Globally Harmonized System